Introduction

Preface:
“Our aim is to provide a Practical, Hands free, Safe, Unique, and 100% Independent Solution for the Visually Impaired”

Information author:
Noureddine El Elani
noor.elelani@student.thomasmore.be
r0708198

Appraisal to all contributors:
Mr. Marc Roggemans

Intro:

Not all people are born equal in their body abilities… With a boost of advanced technology, a help from brilliant programmers, social active helpers, social media, and advertisements, we will be able to assist people with weak abilities, at the moment we’ll be focusing on the visually impaired.

After all, according to the World Health Organization the number of the visually impaired people around the globe is more than 285 million, this big number cannot be reduced by a simple medical prescription, or a magical pill.

It is about time to make their dream a reality, allowing them to live as close as possible, to what we call a standard life.

And that’s why we are building both: a hardware, and a software program, aiming to help the people with little, or no eye sight, in performing their daily activities, freely, safely, 100% hands-free and with no help at all, whether it was a simple activity, such as going out for a cup of coffee with friends, or for even doing their daily full time job, We call it Pathfinder.

Problems and targets:
Some of the problems that I encountered during the semester is trying to work around assembly language, trying to make cheap inaccurate sensors better and more accurate which is impossible the only thing I was able to do is to minimize the error, the problem that I faced the most was receiving defect components or frying components because of vision problems (lesson learnt, triple check before powering the device because my eyes could fool me).

How to detect stairs up/down.

I couldn’t 3d print a module because the school was closed.

Strategy applied for tackling the problem:
By doing a lot of research and staying patient and persistent.

The problem of detecting stairs up or down, I used the two sensors, both sensors have to measure a certain distance to know whether it’s a stairs up or down.

The 3d printed problem was solved by making the module out of a cardboard, not perfect but better than not handing anything in…

Content

Project proposal:

Pathfinder is a fully automated visual aid and navigation system, running in parallel, via sensor-fitted belt in parallel with a smart phone application to provide both unique, and 100% independent experience to the visually impaired, including other features.

Some of the features that will be included in the device is a find/lost system, if the user lost his device somewhere in the room, all they have to do is just clap and the device will start beeping tills it’s moved.

Pathfinder will be the eyes and nerves of the visually impaired, feeding their ears and thus their brain, about their surroundings, to help them deal easily, freely, and safely with what is going on around them.

The device uses 2 ultrasonic sensor (HC-SR04) to detect obstacles such as chairs, tables, people, stairs up, stairs down etc.., the device will be connected to the smartphone application via a Bluetooth module (HC-05), the sound is being detected using a sound sensor (ky-038), an active buzzer is being used for the buzzing sound and an accelerometer (ADXL335) to detect movement, I’m using the Infineon XC888 as the microcontroller of choice.

Hardware Documentation

Block schematic diagram:

Schematic diagram:

PCB layout:

Parts list with pricings (shipping included):

- Infineon XC888 microcontroller: 70€

- Ultra Sonic sensors – HC-SR04 x2: 3 x 2 = 6€

  - https://cdn.sparkfun.com/datasheets/Sensors/Proximity/HCSR04.pdf

- Sound sensor – ky-038: 6€

  - https://datasheetspdf.com/pdf-file/1402048/Joy-IT/KY-038/1

- Bluetooth Module HC-05: 6€

  - https://www.electronicaestudio.com/docs/istd016A.pdf

- Accelorometer – ADXL335: 6€

  - https://www.sparkfun.com/datasheets/Components/SMD/adxl335.pdf

- PCB: 14€

- Active buzzer: 2€

  - http://www.electronicoscaldas.com/datasheet/LTE12-Series.pdf

- Power Bank: 30€

Software documentation

Flowchart:

Listing (source code in ASM):

In the Rapport_PE_tussentijdse.pdf you left me this comment “In your code you state you'll get interrupts every microsecond. This implies your ISR must be executed in under 1 microsecond. The fastest instruction (1 instruction) takes 2x42ns=0.082us, the longest will take 8x42ns=336ns=0,33 microseconds. Your IRS (without you knowing) takes 0.336 microseconds to enter   and   0,252   microseconds   leave.   So without   any   instructions   executed   the   ISR   takes   0,588microseconds!!In the code you test for the echo pin in a loop with a delay of 10 microseconds. This means that you can have a measurement error of 10 microseconds. This gives an error on your measurement of 17 cm.” when I measure using the ultrasonic sensor and I print the results to the LCD screen I get accurate results, I compared some measurements done by the sensor with manual measurements I did using a meter I got the exact same results.

Thank you for the feedback, and sorry about the messy code before...

softch    equ 10h

softcl    equ 11h

confirm   equ 12h

upConfirm equ 13h

downConfirm equ 14h

trig0     bit p3_data.6

echo0     bit p3_data.7

trig1     bit p3_data.4

echo1     bit p3_data.5

inSoundx  bit p3_data.2

org	0000h			      ;origin code in memory

ljmp	start			  ;jump over memory locations containing interrupt vectors

org   000bh			      ;Every interrupt source(s) has a unique end specific vector

ljmp	introutxyz		  ;memory space between vectors is limited. We use the vector

;location to store a ljmp instruction to the actual routine

start:  mov sp,#7fh  			  ;stackpointer initialisation

lcall inits               ;inits

lcall echoinput           ;set p3_data inputs and outputs

lcall startprog           ;timer and ISR inits

loop:   lcall topSensorx          ;top sensor

lcall bottomSensorx       ;bottom sensor

lcall soundSensor         ;sound sensor

ljmp loop

inits:

lcall   XCsw2xtal           ;swtich to crystal oscillator because it's more accurate

lcall	initadc		        ;init

ret

echoinput:

push   syscon0              ;select map

mov    syscon0,#004h        ;addressing the 7 bits

push   port_page            ;save

mov    port_page,#001h      ;select port page 1

mov    p3_pudsel,#05Bh      ;select pull_up device ;01011011

mov    p3_puden,#05Bh       ;select on

mov    port_page,#000h      ;select page 0

mov    p3_dir,#05Bh         ;port 4 as input

pop    port_page            ;retrieve

pop    syscon0              ;retrieve

ret

startprog:

;initialize timer

mov tmod,#00000001b         ;16-bit timer

mov TH0,#0                  ;set timer high to 0

mov TL0,#0                  ;set timer low to 0

mov softch,#00h             ;set softch to 0

mov softcl,#00h             ;set softcl to 0

setb    ET0                 ;interupt0 = 1

setb    EA                  ;enable

clr TF0                     ;clear timer flag 0

ret

topSensorx:

;acceloromter to check if there is movement detected

lcall movement             ;call accelorometer to check if movemnet is detected

cjne a,#1,checkConfirm3    ;compare a = confirm to 1

checkConfirm3:

jnc movDet                 ;if a is = to 1 or bigger that means movement is detected so activate sensor otherwise return

ret

movDet:

;top sensor

jb echo0,skipDelay          ;if echo pin is high skip delay else put trig pin high for 10us

lcall delay10us             ;for 10us

setb trig0                  ;put trig pin high, (works reversed)

lcall delay10us             ;for 10us

clr trig0                   ;then low

skipDelay:

jnb echo0,skipDelay         ;if echo pin is low keep looping else run tr0, wait till echo pin is high

setb tr0                    ;start timer 0

repeat:

;A timeout is not needed because in case the singal didn't return the sensor automatically puts the echo pin low so it will never have an inifinte loop.

jb echo0,repeat             ;if echo pin is high keep looping else stop tr0.

clr tr0                     ;stop timer 0

mov downConfirm,#0          ;move 0 to downConfrim

mov upConfirm,#0            ;move 0 to upConfrim

lcall calc

lcall checkObst             ;osbstacle in front of the user detection using top sesnor

lcall checkobstup           ;use top sensor to check step 1 for stairs up

lcall checkobstDown         ;use top sensor to check step 1 for stairs down

ret

bottomSensorx:

lcall movement             ;call accelorometer to check if movemnet is detected

cjne a,#1,checkConfirm2    ;compare a = confirm to 1

checkConfirm2:

jnc movDet2                ;if a is = to 1 or bigger that means movement is detected so activate sensor otherwise return

ret

movDet2:

;bottom sensor

jb echo1,skipDelay1             ;if echo pin is high skip delay else put trig pin high for 10us

lcall delay10us                 ;for 10us

setb trig1                      ;put trig pin high, (works reversed)

lcall delay10us                 ;for 10us

clr trig1                       ;then low

skipDelay1:

;A timeout is not needed because in case the singal didn't return the sensor automatically puts the echo pin low so it will never have an inifinte loop.

jnb echo1,skipDelay1            ;if echo pin is low keep looping else run tr0, wait till echo pin is high

setb tr0                        ;start timer 0

repeat1:

jb echo1,repeat1                ;if echo pin is high keep looping else stop tr0.

clr tr0                         ;stop timer 0

lcall calc

lcall obstbot                   ;osbstacle in front of the user detection using bottom sesnor

lcall stairsUp                  ;use bottom sensor to check step 2 for stairs up

lcall stairsDown                ;use bottom sensor to check step 2 for stairs down

ret

;soundSesnor

soundSensor:

jnb inSoundx,loopBack           ;if no sound detected don't activate buzzer otherwise do

;buzzer

onSound:

lcall buzzerx                   ;to activate buzzer

;acceloromter

accelorometer:

;lcall movement                 ;call accelorometer to check if movemnet is detected

cjne a,#1,checkConfirm          ;compare a = confirm to 1

checkConfirm:

jnc loopBack                    ;if no movement is detected keep buzzer on otherwise turn off

onSoundBack:

ljmp onSound

loopBack:

ret

;accelorometer

movement:

mov confirm,#0                  ;set confirm to 0

lcall accX                      ;X-axis

lcall accY                      ;Y-axis

lcall accZ                      ;Z-axis

mov a,confirm                   ;set a = to confirm

ret

calc:

mov r3,softch                   ;store mesaured time in registers

mov r2,softcl                   ;idem

mov r1,th0                      ;idem

mov r0,tl0                      ;idem

mov r7,#00h

mov r6,#00h

mov r5,#00h

mov r4,#0Ch                     ;12

lcall div32                     ;divide, result in r4 (time in microseconds and lower than 6553, r3,r2,r1,r0 / r7,r6,r5,r4 Q=r7,r6,r5,r4 R=r3,r2,r1,r0

;r4 contain the timer value in us

mov r1,05h                      ;timer in us

mov r0,04h

mov r3,#00h

mov r2,#3ah                     ;58 = speed of sound in (cm/us)/2 (divided by two because the wave has to go out and come back so we cut in half)

lcall div16                     ;time in us/58

mov r1,03h                      ;result (meter result)

mov r0,02h                      ;result (cm result)

lcall hexbcd16                  ;converts 16 bit hex number to BCD notation

mov softch,#00h                 ;set softch to 0

mov softcl,#00h                 ;set softcl to 0

mov TH0,#0                      ;set timer high to 0

mov TL0,#0                      ;set timer low to 0

ret

;osbstacle in front of the user detection using top sesnor

checkObst:

cjne r1,#0,checkObstBefore

checkObstBefore:

jc noObst                       ;if r1 is less than 0 (0 meters) check second condition else skip,

cjne r1,#1,checkObstafter       ;I know it seems stupid to check below 0 but this is the only way it worked

checkObstafter:

jnc noObst                      ;if r1 is less than 0 (0 meters) check second condition else skip

lcall  inituart1                ;sio and ports, (I had to call uart under and not initialize it above because I'm using the buzzer so they effect each other)

mov    a,#6Fh		            ;send character o = obstacle

lcall  uart1outchar	            ;routine to send character

mov a,#10

lcall delaya0k05s               ;delay between each character to avoid colision

noObst:

ret

;use top sensor to check step 1 for stairs up

checkobstup:

cjne r1,#1,noobstUp             ;if r1 is equal to 1 (1 meter) continue otherwise skip

cjne r0,#20,checkUpobstbefore

checkUpobstbefore:

jc noobstUp                     ;if r0 is equal to or greater than 20 (.20 meters) check second condition else skip

cjne r0,#45,checkUpobstafter

checkUpobstafter:

jnc noobstUp                    ;if r0 is less than 44 (.44 meters) move 1 to upConfirm else skip

mov upConfirm,#1                ;mov 1 to upConfirm, used to make sure

noobstUp:

ret

;use top sensor to check step 1 for stairs down

checkobstdown:

cjne r1,#1,noobstDown           ;if r1 is equal to 1 (1 meter) continue otherwise skip

cjne r0,#0,checkObstDownBefore

checkObstDownBefore:

jc noobstDown                   ;if r0 is equal to or greater than 0 (0 meters) check second condition else skip

cjne r0,#10,checkObstDownAfter

checkObstDownAfter:

jnc noobstDown                  ;if r0 is less than 10 (.10 meters) check second condition else skip

mov downConfirm,#1              ;mov 1 to downConfirm, used to make sure

noobstDown:

ret

;osbstacle in front of the user detection using bottom sesnor

obstbot:

cjne r1,#0,noobstbot             ;if r1 is not equal to 0 (0 meters) skip else continue

cjne r0,#1,checkObstBotBefore

checkObstBotBefore:

jc noobstbot                     ;if r0 is equal to or greater than 1 (.01 meters) check second condition else skip

cjne r0,#90,checkObstBotAfter

checkObstBotAfter:

jnc noObstBot                    ;if r0 is less than 90 (.90 meters) send character else skip

lcall  inituart1                 ;sio and ports

mov    a,#6fh		             ;send character u = stairs up

lcall  uart1outchar	             ;routine to send character

mov a,#10

lcall delaya0k05s                ;delay between each character to avoid colision

noObstBot:

ret

;use bottom sensor to check step 2 for stairs up

stairsUp:

cjne r1,#1,noUp                     ;if r1 is not equal to 1 (1 meters) skip else continue

cjne r0,#0,checkUpAfter

checkUpAfter:

jc noUp                             ;if r0 is less than 0 (.0 meters) check second condition else skip

cjne r0,#5,checkUpAfter0

checkUpAfter0:

jnc noUp                            ;if r0 is less than 5 (.05 meters) send character else skip

mov a,upConfirm                     ;move upConfirm to a

cjne a,#1,noUp                      ;if upConfirm = 1 send character else skip

lcall  inituart1                    ;sio and ports

mov    a,#75h		                ;send character u = stairs up

lcall  uart1outchar	                ;routine to send character

mov a,#10

lcall delaya0k05s                   ;delay between each character to avoid colision

noUp:

ret

;use bottom sensor to check step 2 for stairs down

stairsDown:

cjne r1,#3,checkDownBefore0

checkDownBefore0:

jc noDown                            ;if r1 is equal to or greater than 3 (3 meters) check second condition else skip

cjne r0,#90,checkDownBefore

checkDownBefore:

jc noDown                            ;if r0 is equal to or greater than 90 (.90 meter) check downConfirm else skip

mov a,downConfirm                    ;move downConfirm to a

cjne a,#1,noDown                     ;if downConfirm = 1 send character else skip

lcall  inituart1                     ;sio and ports

mov    a,#64h		                 ;send character d = stairs down

lcall  uart1outchar	                 ;routine to send character

mov a,#10

lcall delaya0k05s                    ;delay between each character to avoid colision

noDown:

ret

;x-axis of acceloromoter

accX:   lcall	initadcf8	                  ;initialise ADC for basic usage (only getadc4to7 can be used) overclocked 10 bit resolution

mov	a,#10000000b	                  ;select p2.7

lcall	getadc4to7	                  ;measure

cjne a,#60h,checkXBefore

checkXBefore:

jc noX                                ;if a is less than 60h add 1 to confirm else check second condition

cjne a,#70h,checkXAfter

checkXAfter:

jc skipX                              ;if a is equal to or greater than 70h add 1 to confirm else skip

noX:    mov a,#1

mov b,confirm

add a,b

mov confirm,a

skipX:

ret

;y-axis of accelorometer

accY:   lcall	initadcf8	                   ;initialise ADC for basic usage (only getadc4to7 can be used) overclocked 10 bit resolution

mov	a,#01000000b	                   ;select p2.6

lcall	getadc4to7	                   ;measure

cjne a,#60h,checkYBefore

checkYBefore:

jc noY                                  ;if a is less than 60h add 1 to confirm else check second condition

cjne a,#70h,checkYAfter

checkYAfter:

jc skipY                                ;if a is equal to or greater than 70h add 1 to confirm else skip

noY:    mov a,#1

mov b,confirm

add a,b

mov confirm,a

skipY:

ret

;z-axis of accelorometer

accZ:   lcall	initadcf8	                     ;initialise ADC for basic usage (only getadc4to7 can be used) overclocked 10 bit resolution

mov	a,#00100000b	                     ;select p2.5

lcall	getadc4to7	                     ;measure

cjne a,#70h,checkZBefore

checkZBefore:

jc noZ                                   ;if a is less than 70h add 1 to confirm else check second condition

cjne a,#80h,checkZAfter

checkZAfter:

jc skipZ                                 ;if a is equal to or greater than 80h add 1 to confirm else skip

noZ:    mov a,#1

mov b,confirm

add a,b

mov confirm,a

skipZ:

ret

;buzzer

buzzerx:

lcall   initlcd                           ;initailize lcd for buzzer, lcd must be initialized, because uart will interupt the buzzer port

lcall	lcdbuzon			              ;buzzer on

mov	a,#12

lcall	delaya0k05s			              ;delay

lcall	lcdbuzoff	                      ;off

mov	a,#12

lcall	delaya0k05s                       ;delay

ret

inituart1:

lcall	ports		                       ;set port

lcall	baudrate	                       ;set baud rate

ret

; port p0.5 as input

; poort p0.4 as output

ports:  push	port_page	                       ;backup port_page state

mov	port_page,#00h	                       ;select page 0

mov	p0_dir,#00010000b                      ;p0.4 = output

mov	p0_data,#ffh                           ;all outputs on one

mov	port_page,#02	                       ;select page 2

mov	p0_altsel0,#00010000b                  ;altsel for p0.4

mov	p0_altsel1,#00010000b                  ;idem

pop	port_page		                       ;retrieve port_page old state

ret

baudrate:

lcall	mapregs

mov    scon1,#01010000b                    ;initialize UART

;first initialize BG, then bcon, otherwise BG won't be used

mov    bg1,#155    		                   ;don't use fraction divider

mov    bcon1,#00000001b	                   ;ativate baud rate generator

mov    fdcon1,#00000000b                   ;fractional divider is off

lcall  nomapregs

ret

uart1outchar:

lcall	mapregs	                           ;regs of uart1 sit in the mapped zone

MOV    SBUF1,A                             ;send character

uart1outchar1:

JNB    scon1.1,uart1outchar1               ;wait till sender is available

;CLR    scon1.1                            ;clear bit

;***** clr scon1.x instruction because of a bug in the chip!!

;***** we solve it using anl scon1,#11111101b

anl	scon1,#11111101b                       ;alternative for clear TI flag

lcall	nomapregs	                       ;back to standard set regs

ret

introutxyz:	                                       ;subroutine

;no infinite loops in interupts

push acc

push psw

;everytime there's an overflow an interupt occurs thus inc 1

;16 bit addition

clr tf0                                    ;clear timer flag 0

mov a,softcl                               ;mov softcl to a

add a,#1                                   ;add 1 to a

mov softcl,a                               ;write low result back

mov a,softch                               ;high byte addition, use carry

addc a,#0

mov softch,a                               ;write high result back

pop psw

pop acc

reti                                       ;end interrupt routine

#include	"c:\xcez4.inc"

User manual:

Poster:

Decision:

Was the target met?
Yes and no, it’s functional most of the time but the ultrasonic sensors are so volatile, the solution to solve this problem is to use lidar sensors or object detection(which is way out my knowledge), and the lidar sensors are out of my budget, so as a proof of concept/prototype, I would say yes, the target was met.

What would you do differently next time?

- Order components earlier

- Have a plan b in case of a pandemic

- split the time spent on the project instead of spending a lot of time on it at once (time management)

- Ask for help when stuck, instead of ruining the code more.

- Don’t underestimate a project

- Don’t underestimate the amount of time needed

- Hardware doesn’t forgive mistakes like software

- Don’t work 16 hours straight because after 8 hours straight work, you’ll end up losing productivity and focus, and that will occur in more problems.

Reflection over the total cost: Compare with original budget.
Shipping costs got a bit out of hand, because we couldn’t split the shipping costs, but other than that it was close enough.

Reflection to market: Where is your approach different/better/worse?
This solution is better than what’s available on the market because it uses technology which far better and more accurate than the old outdated canes, of course the device  currently is just a prototype, but suppose it’s a finalized ready to go to market device I can 100% say a lot better.

Gantt Chart:

Github:

https://github.com/noorilpsp/pathfinder.git